Gesture Recognition Using Deep Learning-Based Single-Pixel Imaging

Hiroki Takatsuka, Shiro Suyama and Hirotsugu Yamamoto

# Utsunomiya University, Tochigi, Japan

# With the recent advancements in information and communication technology, information displays have become popular in our daily lives. By combining these information displays with gesture recognition technology, it becomes possible to create interactive information interfaces that can switch images on the display based on the user's gestures. However, using cameras capable of capturing detailed images for gesture recognition is not feasible in many places due to concerns regarding privacy. To address this problem, we have proposed a method of capturing shadow pictures using single-pixel-imaging (SPI) to realize privacy-conscious gesture recognition. SPI is a technique that utilizes spatially modulated illumination and a single light detector to capture images. It allows imaging under low-light conditions and with light sources other than visible light, making it applicable in a wide range of scenes.

# In this presentation, we introduce an overview of gesture recognition using single-pixel imaging with deep learning. Furthermore, we explain a method for making latent illumination patterns in signage images, which enables the use of existing public displays as illumination sources while preserving privacy, to remove the influence of apparent images using U-Net, and for reconstructing and classifying gesture images using deep learning. We also explain measurements of single-pixel imaging using an aerial display as a light source, and a technique for measuring 3D positions with a single pixel detector, where the distance is calculated from the disparity caused by shifting aerial illumination patterns.

# These studies contribute to the realization of privacy-conscious gesture-recognition sensors that do not rely on imaging cameras. By utilizing existing digital signage and aerial displays as illumination patterns, the proposed approach can be implemented in public spaces without additional optical components. This approach opens new possibilities for interactive public displays and aerial display.

# Short biography:

# Hiroki Takatsuka received his B. E. and M. E. degrees from Utsunomiya University, Japan. Currently, he is a Ph. D. candidate at Utsunomiya University, Japan. His research interests include deep-learning assisted single-pixel-imaging and ultra-wide field-of-view display using aerial image.

# Author’s Publications / Presentations

- Takatsuka H, et al. Absolute parallax and reconstruction position of aerial image formed just behind viewing eyes in ultra-wide field-of-view display. ITE Trans Media Technol Appl. 2025;13(1):8–13.

- Takatsuka H, et al. Ultra-Wide Field-of-View Display by Use of Aerial Stereoscopic Images Reconstructed Just Behind the Eyes. Proc. IDW’23 2023;30: 1553–1556. Best Student Paper Award.

- Takatsuka H, et al. Improvement of Reconstruction Efficiency for Gesture Recognition by Single-Pixel-Imaging Using Apparent Image with Latent Random Pattern for Each RGB Color Plane. In Information Photonics 2023 2023;24. Best Student Paper Award.

- Takatsuka H, et al. Removal of apparent images using U-Net in single-pixel-imaging with high-frame-rate display with latent random patterns. J Jpn Soc Precis Eng. 2024;90(5):426–430. Japanese.

- Takatsuka H, et al. Gesture recognition using deep-learning in single-pixel-imaging with high-frame-rate display with latent random dot patterns. Opt Rev. 2023;31(5):116–125.

- Takatsuka H, et al. Elimination of Apparent Image on Single-Pixel-Imaging by Use of High-Frame-Rate Display Using Latent Random Dot Patterns with U-Net., In the 12th Korea–Japan Workshop on Digital Holography and Information Photonics 2022;49. Student Presentation Award.